Table of Contents

# EEG-CAR-01 Rev C – work report

Not a controlled document. It is the report on the work of 8 September 2026 that ECO-EEG-030 to ECO-EEG-033 record, and it says what was run, what the repository contradicted, what was assumed, and what is still open. Licence: CC BY-SA 4.0

## 1. What was run, and what it printed

### 1.1 Baseline, on the untouched tree

| Command | Result |
| --- | --- |
| tools/emit_all.py --cached | run. 0 violations, 145 of 145 nets connected, 3 745 segments, 552 vias, minimum clearances 0.260 / 0.285 / 0.285 / 0.275 mm. Every text output reproduced byte for byte against the committed files; only the PNG, PDF and zip outputs differed, on embedded timestamps |
| tools/simulate_production.py | run. 186 passed, 0 failed, 5 open – not the 193 / 0 / 6 the register stated. See 2.1 |
| kicad-cli --version | NOT RUN. KiCad is not installed on this machine and could not be installed. No ERC and no DRC result is claimed anywhere in this work |

Four Python packages had to be installed before anything would run at all: shapely, scikit-image, numpy (present) and trimesh. cadquery could not be installed: the machine had 472 MB of free disk and cadquery-ocp needs several times that. tools/mech_gen.py imports cadquery at module scope, so it cannot even be imported here, and no printed part was regenerated in this work.

### 1.2 The Rev B geometry under the new rule set

tools/grade_revb.py, on the released kicad/EEG-CAR-01_RevB_routed.kicad_pcb and tools/routed.pkl, census checked first (3 745 segments, 552 vias):

| Finding | Rule | Count |
| --- | --- | --- |
| 1 | vias inside an SMD pad | 67 |
| 2a | dangling track ends | 5 |
| 2b | redundant copper loops outside a plane | 221, on 22 nets |
| 3 | angle below 90 degrees between two segments | 171 |
| 3 | segments off a 0/45/90/135 degree axis | 158 |
| 4 | off-centre pad entries | 299 |
| 5 | conductors under their class minimum | 322 |
| 5 | net classes on a forbidden layer | 50 |
| 5 | vias on a class that forbids them | 18 |
|  | total | 1 311 |

The Rev B DRC report is not withdrawn. It measured the twelve rules it had, correctly.

Two counts in tools/drc_geometry.py were wrong before they were right, and both are recorded in the source rather than quietly fixed. The loop check first modelled copper as touching polygons and reported 3 356 loops, where three mutually touching polygons are not a loop; re-modelled topologically – junctions as nodes, segments as edges, T-junctions split – it reported 476; excluding degenerate edges, where a segment’s two ends land on the same via, it reports 221. The first two numbers are wrong and are not quoted anywhere else.

### 1.3 The schematic against the design source

tools/sch_netlist.py parses the emitted .kicad_sch files back – pin geometry from their own embedded lib_symbols, wires, junctions, labels, hierarchical labels, power symbols – and diffs. It does not ask the generator what it meant.

|  | design.py | schematic |
| --- | --- | --- |
| nets | 156 | 156 |
| pins | 614 | 614 |
| structural problems | – | 0 |
| netlist differences | – | 0 |

It found four real errors on its first run: R50, R51, R52 and R94 were wired by geometry rather than by pin number and had pin 2 where design.py has pin 1, and R94/R95 were drawn as a divider instead of two I2C pull-ups. A resistor is symmetric, so the circuit was right and the netlist was wrong. Both are fixed and the helper that caused it now wires by pin number.

### 1.4 ERC, DRC and the collision check

| Check | State |
| --- | --- |
| kicad-cli sch erc | NOT RUN. KiCad is not installed |
| kicad-cli pcb drc | NOT RUN. Same |
| structural ERC (tools/sch_netlist.py) | run: 0 findings. Every pin on a net; no designator placed twice and every unit of a multi-unit part placed once; no accidental single-pin net; every power net driven by a power symbol |
| board read-back (tools/emit_revc.py) | run: passes. 211 footprints, 636 pads, 156 nets, 0 segments, 0 vias, 37 locked, netlist identical to design.py, every rule area present under the name the .kicad_dru uses, project classes and all 156 assignments complete, track_dangling at error |
| footprint audit (tools/footprint_audit.py) | run: 183 PASS, 0 FAIL, 3 OPEN, 0 UNRECOGNISED, 25 fabricated features over all 211 designators |
| collision check (tools/collision_check.py) | run: 15 findings. Three of its four cases are open. See 1.5 |
| .kicad_dru bracket check | run: 37 rules, balanced |
| STEP structural check | run: all files closed, no dangling entity reference. No STEP reader was available, so no CAD kernel has opened them |
| tools/simulate_production.py, after | run: 193 passed, 0 failed, 7 open |

### 1.5 The collision check, in full

| Case | Result |
| --- | --- |
| A – carrier parts against the plate underside | clear. Tallest is J15 at 10.00 mm against a plate underside at 18.0 mm |
| B – the DevKit against the plate opening | OPEN. The opening is 2 mm too short. J6/J7 are 1x22, so the pin span is 53.34 mm and the ESP32-S3-DevKitC-1 PCB is 63.0 mm: it needs carrier Y 3.2 to 66.2 mm and MP-01’s opening runs 4.0 to 65.0 mm |
| C – modules against each other and jumper reach | OPEN. Six of twelve cannot be placed, and four cannot reach their connector within the 60 mm of ICD-EEG-006 section 3.2 from any position on the plate – M4 to J10 is 86 mm at best, M5 to J11 is 72 mm at best. That 60 mm is a crosstalk limit |
| D – plan area | OPEN. 17 356 mm2 of modules into 12 409 mm2 of plate: 140 % fill |

Case D does not rest on the assumed envelopes. The two ADS1299 boards alone are 7 345 mm2, 59 % of the net usable plate, and that figure is the Raspberry Pi HAT mechanical standard rather than a guess: the PiEEG-8 is a Pi shield. Halving every other envelope still leaves 12 351 mm2 against 12 409 mm2.

### 1.6 Reproducibility

Two consecutive runs of tools/emit_all.py produce byte-identical SHA-256 checksums across all 51 files of the handover set. Every uuid in the schematic is uuid5 over a fixed namespace, so a regeneration is diffable against the last one.

## 2. Where the repository contradicted the brief

| # | The brief said | The repository says |
| --- | --- | --- |
| 1 | the simulator’s last run was 193 passed, 0 failed, 6 open | On this machine the untouched tree printed 186 / 0 / 5. Six checks need cadquery and one open item needs trimesh; installing trimesh gave 187 / 0 / 6, and cadquery could not be installed. The register’s 193 was obtained on a machine with cadquery. The run behind this work prints 193 / 0 / 7 and is not the same 193: six checks were added and six lost, and they cancel. A machine with cadquery would print 199 |
| 2 | “four op-amp packages moving from SOIC to TSSOP” | Three. U1, U2 and U3. There is no U4; U7 is the SOT-23-5 comparator |
| 3 | the Murata part number should be corrected “if it does not encode 600 Ω” | The part number does encode 600 Ω – Murata’s own numbering page reads the three figures as two significant digits and a count of zeros, so 601 is 600 Ω. The reviewer’s reading was wrong. The defect is elsewhere and is real: PG is the large-current series and it is not made at 600 Ω in 1608, whose range stops at 470 Ω. BLM18PG601SN1D is not a part. Corrected to BLM18AG601SN1D |
| 4 | three BOM errors | Four. Omron B3F-4055 is a 12 x 12 mm switch on a 6 x 6 mm land pattern, and at 260 gf it also misses AVL-EEG-017’s own 160 ± 50 gf window. Found by the audit the brief asked for, not by any reviewer. Corrected to B3F-1052 |
| 5 | ICD-EEG-006 section 1 is the one home of the connector table | Four of its coordinates disagreed with design.py: J4 (41.0, 36.0 against 41.0, 50.0), J9 (66.0, 78.0 against 114.0, 62.0), J22 (30.0, 116.0 at 90° against 15.7, 73.5 at 270°) and J26 (128.0, 104.0 against 117.0, 8.0). Two moved at ECO-EEG-018 and the table never followed. design.py governs; the table is corrected and RUL-EEG-021 ruling 6 is added |
| 6 | design.py carries 68 kΩ (ECO-EEG-024) | The value did. design.py’s NOTES, which are printed onto the fabrication and assembly drawings, still said 47 kΩ; also said the analogue rails arrive at “J2 pins 11-14”, where J2 has ten ways and they arrive at J23; and said the keep-outs are clear “on both layers” on a four-layer board. All three corrected |
| 7 | emit kicad/EEG-CAR-01_RevC_unrouted.kicad_pcb and EEG-CAR-01_RevC.kicad_pro | Those two names are incompatible. A KiCad project expects its board to share the project’s stem; a differently named board opens standalone, without the net classes, the 156 assignments or the DRC severities. The board is EEG-CAR-01_RevC.kicad_pcb. See open question 1 |
| 8 | the rule sheet is one page | It is three. See open question 2 |
| 9 | five release-state paragraphs say “released for review” | Sixteen documents did, sixty-two occurrences. The five named are corrected, and so are the other six live statements in the four documents this round re-issued. Eight documents it did not re-issue still carry it. See open question 3 |
| 10 | the DRC report is the authority for the routed result | It is, and it is authoritative about the rule set it had. RUL-EEG-021 ruling 5 now says that a DRC report states the rule set it measured against |

## 3. Assumptions

Every one is registered or flagged. None is a sign-off.

| # | Assumption | Where it is registered | Confidence |
| --- | --- | --- | --- |
| 1 | The twelve module envelopes, and their placement on MP-01 | ASM-EEG-023 Rev B, MECH-D6-MODULE-ENVELOPES, new and blocks a build | judgement. No vendor drawing was consulted for any of them. Two are form-factor derived: the PiEEG-8 is a Pi HAT (65.0 x 56.5 mm) and the Pololu S13V15F5 is a 0.5 x 0.8 inch board |
| 2 | ELECTRODE minimum width 0.25 mm, above the board’s 0.20 mm floor | tools/rules.py, flagged confirm; printed in LAY-EEG-034 section 5 | judgement. Open question 4 |
| 3 | ELECTRODE on L1 only, no vias | same | DSN-EEG-003 section 3.3 rule 3 read literally. Open question 4 |
| 4 | ANALOGUE_REF minimum width 0.30 mm | same | impedance, not current. Open question 4 |
| 5 | Pad entry within 25 % of the pad’s minor dimension | tools/rules.py; LAY-EEG-034 section 6 | judgement: finding 4 has no number in it |
| 6 | The schematic sheet split, nine sheets | tools/emit_kicad_sch.py, checked to be a partition of design.C | judgement |
| 7 | KiCad 8 – (kicad_sch 20231120), (kicad_pcb 20240108) | the handover README, with an offer to re-emit | assumed, as the brief instructed. Open question 5 |
| 8 | Carrier component heights per footprint class | tools/mech_bodies.py, each with its source | datasheet, except the DIN 42802 socket at 10.00 mm, which has no qualified part |
| 9 | The DevKit stands 13.6 mm and its PCB is 63.0 x 25.5 mm | ICD-EEG-006 section 4 (calculated) and Espressif’s published outline | derived |

## 4. Open questions

1. The board file name. The brief asked for EEG-CAR-01_RevC_unrouted.kicad_pcb beside EEG-CAR-01_RevC.kicad_pro. A KiCad project expects board and schematic to share its stem, and a board opened standalone loses the eight net classes, the 156 assignments and the severity that makes track_dangling an error – most of the specification. Recommended default, and what is implemented: the board is EEG-CAR-01_RevC.kicad_pcb, and that it is unrouted is said in its title block, its comments, LAY-EEG-034 and the README.

2. The rule sheet is three pages, not one. Compressing it means dropping the derivation of each net-class number, and a rule whose reason is invisible is a rule that gets relaxed at the first difficulty. Recommended default: leave it at three. If one page is needed for the order paperwork, sections 5 and 6 alone fit on one and the rest becomes an annex.

3. Eight documents still say the data is “released for review”. ASM-EEG-007, PARTS-EEG-019, QP-EEG-010, REG-EEG-012, RFQ-EEG-001, RISK-EEG-011, SVC-EEG-013 and TST-EEG-004. Three of them are out of scope by instruction (RISK-EEG-011, REG-EEG-012 and the research documents), and RFQ-EEG-001 additionally carries E-26’s self-contradicting switch class. Recommended default: one sweep ECO covering the five that are not out of scope, plus the RFQ at its next issue. It is mechanical and it is the defect RUL-EEG-021 section C ruling 4 forbids.

4. Four net-class numbers need your confirmation before the order goes out, because they constrain the contractor and they are this programme’s proposals, not transcriptions: the ELECTRODE minimum width of 0.25 mm; ELECTRODE restricted to L1 with no vias at all; the ANALOGUE_REF minimum width of 0.30 mm; and the 25 % pad-entry tolerance. Recommended default: confirm all four. The electrode restriction is the one worth a second look – it is DSN-EEG-003 section 3.3 rule 3 read literally and it is achievable on the present placement, but if the contractor says it forces a respin, relaxing it is a decision and not a concession to be made silently.

5. Which KiCad version is JLCPCB on? Everything is emitted for KiCad 8. Re-emitting for another major version costs nothing because it is generated. Recommended default: ask Lyle He before the order and re-emit if the answer is not 8.

6. MP-01 cannot carry twelve modules. 140 % fill, and the plate’s DevKit opening is 2 mm short. Four routes would each move it – a larger plate; a second tier of standoffs; an ADS1299 breakout that is not a Pi HAT, which AVL-EEG-017 section 2 M1 permits under qualification even though the device may not change; or moving the modules whose jumpers cannot reach onto the carrier as fitted parts. None is decided and none is costed. Recommended default: buy one of each module and measure it before deciding anything, because ten of the twelve envelopes are assumptions. The two that are not – the ADS1299 boards – are 59 % of the plate on their own, so the measurement most likely to settle it is the PiEEG-8’s actual outline.

7. Should the in-house router produce a reference route for comparison? It was not done: the brief puts it last and everything ahead of it took the time. Recommended default: no, until the contractor’s placement is in. A reference route against a placement nobody has confirmed compares two guesses.

8. firmware/main/board_pins.h is still hand-maintained against a check in tools/simulate_production.py. Untouched here, as instructed; no pin assignment moved, so it did not need to move. Recommended default: leave it until a change forces a pin move, then generate it.

## 5. The handover set

kicad/RevC_layout_inputs/ – 51 files, 2.6 MB, with SHA256SUMS.txt carrying a SHA-256 and a one-line description for each. The manifest is the authority and is not reproduced here.

| Group | Files |
| --- | --- |
| project | EEG-CAR-01_RevC.kicad_pro |
| schematic | EEG-CAR-01_RevC.kicad_sch + nine sheets, EEG-CAR-01.kicad_sym |
| board | EEG-CAR-01_RevC.kicad_pcb, EEG-CAR-01_RevC.kicad_dru |
| netlist and BOM | EEG-CAR-01_RevC-IPC-D-356A.ipc, EEG-CAR-01_RevC_BOM.csv, footprint_audit_RevC.md |
| placement | two provisional CPLs, EEG-CAR-01_RevC_outline_and_fixed_connectors.dxf |
| 3D | 22 footprint bodies under step/footprints/, three assembly bodies under step/ |
| evidence | EEG-CAR-01_RevC_schematic_netlist_check.txt, EEG-CAR-01_RevC_collision_check.txt |
| rules | LAY-EEG-034_RevA_carrier_layout_rule_sheet.pdf and .md |
| covering note | README_layout_inputs.md, SHA256SUMS.txt |

Nothing was sent. No tool in tools/ transmits anything.

## 6. Commits

| Commit | Work package |
| --- | --- |
| c652258 | WP1 revision bookkeeping and the Rev B withdrawal ECO (ECO-EEG-030) |
| 34feea1 | WP2 BOM and footprint corrections (ECO-EEG-031) |
| 553bec1 | WP3 the seven findings encoded as rules, and Rev B regraded (ECO-EEG-032) |
| e15724c | WP4 the native KiCad schematic (ECO-EEG-033 part 1) |
| b142243 | WP5 unrouted board, project file and DXF (part 2) |
| 9e96ce1 | WP6 3D bodies and the collision check (part 3) |
| 0fb4e9d | WP7 LAY-EEG-034, the layout rule sheet (part 4) |
| 4b87f44 | WP8 the handover set and its README (part 5) |
| 385429a | WP9 document register close-out |
| 17c02ad | WP9 (cont) release-state statements in the four re-issued documents |

Documents advanced a letter: DSN-EEG-003 to D; ECO-EEG-016, ICD-EEG-006, JIG-EEG-009 and AVL-EEG-017 to C; FW-EEG-001 to D; ASM-EEG-023 and RUL-EEG-021 to B. LAY-EEG-034 is new at Rev A. 218 citations and 24 file names followed, by tools/revision_bump.py.

Nothing in this package has been manufactured or measured. No safety engineer has reviewed this design. No layout engineer has seen Rev C, because Rev C has no layout yet.